	MTI Quad-LNB         €  70.-
	


	
ArtNr.: 1028
	
	


	mit eingebautem Multischalter für 4 Receiver
0,2 dB typ.
40 mm Feed
Wetterschutzgehäuse

Eigenschaften
RF Input
Input Frequency Range Low Band: 10.7 ~ 11.70 GHz
Input Frequency Range High Band: 11.7 ~ 12.75 GHz

IF Input
Output Frequency Range Low Band: 950 ~ 1950 MHz
Output Frequency Range High Band: 1100 ~ 2150MHz
Output Connector Type: 75 Ohm Female Connector
Output VSWR: 2.0:1 (Typ.) @ Room Temperature

Local Oscillator
LO. Frequency Low Band: 9.75 GHz High Band: 10.6GHz
LO. Frequency Stability ±1MHz (Max.) @ Room Temperature
LO. Frequency Phase Noise 
-55dBc/Hz@ 1KHz (Max.)
-80dBc/Hz@10KHz (Max.)
-100dBc/Hz@100KHz (Max.)
-105dBc/Hz@1MHz (Max.)
-110dBc/Hz@10MHz (Max.)

Conversion Performance @Room Temperature
Conversion GAIN: 62dB (Max.)
Gain Variation (P-P): 4dB (Typ.)
Gain Flatness (across any 26 MHz segment): ± 0.5dB (Typ.)
Noise Figure Low Band: 0.2dB (Typ.)
High Band: 0.3dB (Typ.)
Image Rejection: 45dB (Min.)
Cross Polarization: 25dB (Typ.)

Power Requirements
DC Current Consumption: 250mA (Typ.)
Operating Voltage Horizontal: 16 ~ 19 Vdc
Operating Voltage Vertical: 11,5 ~ 14 Vdc
Band Switching Low Band: 0 Hz
Band Switching High Band: 22 KHz ± 4KHz

Environmental
Operating Temperature Range -40°~ +60°
Storage Temperature Range -55°~ +80°
	


